Cottage Food Operation  (CFO) Water Source

Water used for cottage food operations must be from an approved source. The California Health and Safety Code, Section 113869, requires water from a private well, spring, or surface water to comply with the standards for transient noncommunity water systems in the California Safe Drinking Water Act. Water must be tested as shown in the chart below. Sample collection must be performed by personnel trained to collect samples by a certified lab or a certified operator.

Analysis type							Frequency

Bacteriological							Quarterly

Nitrate/Nitrite

Nitrate (as NO3)							Yearly

Nitrite (as N)							Every 3 years

Inorganics

Fluoride								Once

Secondary Standards

Aluminum							Once

Color								Once

Copper								Once

Corrosivity-aggressive index					Once

Foaming Agents (MBAS)						Once

Iron								Once

Manganese							Once

Methyl-tert-butyl-ether (MTBE)					Once

Odor-threshold							Once

Silver								Once

Turbidity							Once

Zinc								Once

Total Filterable Residue (TDS)					Once

Specific Conductance (E.C.)					Once

Chloride								Once

Sulfate								Once

General Mineral

Bicarbonate 							Once

Carbonate 							Once

Total Alkalinity 							Once

pH								Once

Calcium								Once

Magnesium							Once

Sodium 								Once

Total Hardness 							Once

Volatile Organic Compounds	Need is based on distance from potentially comtaminating activities