University of California

Cooperative Extension

Assessing the Condition of Your Roads

The purpose of this worksheet is to enable you to evaluate the condition of the roads on your property. Understanding road conditions will help you plan and prioritize actions that will improve your ability to manage your property. The assessment process includes several steps:

- Identifying access routes to your property and your rights and responsibilities concerning them.

- Mapping existing roads on your property.

- Deciding which roads you need for management.

- Deciding if you need new roads.

- Evaluating the condition of your existing roads, including both those you will use and those you won’t use.

Note: The worksheet asks for yes-no answers. A don’t know answer may imply a need-to-know.

# Access to Your Property

If a public road adjoins or abuts your property and you have a legal encroachment permit to use it for access, you are in good shape.  However, just because you have a driveway from a public road to your property does not necessarily mean that it is legal. Check your deed or check with the jurisdiction (city, county or state) that has responsibility for the public road. If you have access from a road on land owned by a federal agency such as the Forest Service, check with the agency’s local offices.

If you have access to your property through another property or by private road, you need to know if you have legal rights of access. This is usually recorded on your deed as a right-of-way. If you don’t have deeded access, then you need to know what rights you have to use the road. You also need to know what responsibilities you have for maintaining the road and what uses are restricted (e.g., hauling logs). In the absence of a legal right to use the road, you are vulnerable to the whims of your neighbor and need to negotiate that right.

- Does a public road provide access to your property (e.g., county road or state highway)?

  Yes	 No 		 Don’t Know

- Does a private (including public agency) road provide access to your property (e.g., subdivision road, road through adjoining private or public lands)?

  Yes	 No 		 Don’t Know

- Do you have deeded access to your property?

  Yes	 No 		 Don’t Know

If you don’t have deeded access, by what right can you use the road?

- If access is by private road, does it receive regularly scheduled maintenance?

  Yes	 No 		 Don’t Know

- Are you part of a road maintenance agreement or association?

  Yes		       No 		 Don’t Know

- Do you have responsibilities for maintaining roads that provide access to your property?

  Yes	 No 		 Don’t Know

# Access on Your Property

There may be a number of roads on your property, some of which are a legacy of past uses (e.g., old logging roads). Legacy roads can be a liability if they are causing environmental damage. Sometimes roads are shown on published topographic maps or other maps of your property. Sometimes they need to be located in the field or with aerial photographs and sketched on a map.

- To the best of your ability, use a topographic map (quadrangle) to indicate the road system on your property. Identify and number (Road #1, Road #2, etc.) main and secondary roads that are in use as well as any roads that exist but are not used.

- Does your on-site road system allow you to access all of your property?

  Yes	 No 		 Don’t Know

- Indicate on a map what parts of your property are not currently accessible by road.

- Now, you need to walk or drive your roads. For each existing road, provide the following descriptive information (note: for all these tables, use the road numbering system for reference and duplicate tables for as many roads and crossings as necessary):

|  | Road #1 | Road #2 | Road #3 |
| --- | --- | --- | --- |
| Width (running surface in feet) |  |  |  |
| Range of grades (from steepest to gentlest slopes, in percent) |  |  |  |
| Surfacing (native, rock, paved) |  |  |  |
| Drainage (insloped with inboard ditch, outsloped, not evident or don’t know) |  |  |  |

- For each existing road, provide any information on the following problems. If possible, use your map to indicate where the problems are located:

|  | Road #1 | Road #2 | Road #3 |
| --- | --- | --- | --- |
| Cutback sloughing or failure |  |  |  |
| Fill slope erosion or failure |  |  |  |
| Rutting on the road surface |  |  |  |
| Potholes on the road surface |  |  |  |
| Wet spots on the road surface |  |  |  |
| Gully erosion below the road |  |  |  |
| Eroding ditch or plugged cross drains |  |  |  |
| Other |  |  |  |

- Indicate on a map all existing stream crossings on existing roads (note: streams may be ephemeral, intermittent or perennial). Identify each by sequentially numbering in relation to road numbering system (Road #1, Crossing #1, Road #2, Crossing #1, etc.).  For each crossing, provide the following descriptive information:

| ## Road #____ |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Crossing # ___ | Crossing # ___ | Crossing # ___ | Crossing# ___ |
| Crossing type (culvert, ford, bridge) |  |  |  |  |
| Crossing condition (good, fair, poor) |  |  |  |  |
| Fill slope condition (stable, some erosion, major erosion) |  |  |  |  |

- For each crossing, provide any information on the following problems:

| ## Road #____ |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Crossing # ___ | Crossing # ___ | Crossing # ___ | Crossing# ___ |
| Crossing undersized for the size of the stream |  |  |  |  |
| Crossing not aligned with channel |  |  |  |  |
| Crossing is not at the slope of the stream |  |  |  |  |
| Downstream channel or bank erosion |  |  |  |  |
| Upstream channel or bank erosion |  |  |  |  |
| Upstream sediment deposition |  |  |  |  |
| Prone to plugging with debris |  |  |  |  |
| Diversion potential |  |  |  |  |

Summary of Findings

- Is access to your property adequate?

  Yes	  No 		  Don’t Know

- If not, what issues need to be addressed?

  - Rights of access:

  - Road condition and maintenance:

  - Other:

- Is access within your property adequate?

  Yes	  No 		  Don’t Know

- If not, what issues need to be addressed?

- Need for additional access:

- Condition of roads that are used or will be used:

- Condition of other roads:

- Condition of crossings on used roads:

- Condition of crossings on unused roads: